|  | Simulation of AutoRecover Of ShelledModel1 Date: March 30, 2023 Designer: Solidworks Study name: Static 5 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 11. |
| --- | --- |
| # Description Torsional Forces Analysis. Torsional Forces were applied along all 4 columns to simulate the forces the subframe would typically experience |  |

# Assumptions

# Model Information Model name: AutoRecover Of ShelledModel1 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror4; Solid Body; Mass:3.3 kg Volume:0.00111137 m^3 Density:7,847.94 kg/m^3 Weight:32.24 N; C:\Users\westy\OneDrive\Documents\SubFrame Documents\Frame_Documents\ShelledModel1.SLDPRT Mar 30 11:46:53 2023.

# Study Properties Study name; Static 5. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\westy\OneDrive\Documents\SubFrame Documents\Frame_Documents).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/mm^2 (MPa).

# Material Properties Model Reference; Properties; Components. Name:; AISI 4130 Steel, normalized at 870C. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 460 N/mm^2. Tensile strength:; 731 N/mm^2. Elastic modulus:; 205,000 N/mm^2. Poisson's ratio:; 0.285. Mass density:; 7.85 g/cm^3. Shear modulus:; 80,000 N/mm^2.; SolidBody 1(Mirror4)(AutoRecover Of ShelledModel1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 4 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.20706; 0.00578785; -14.8168; 14.8183. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Torque-1; Entities:; 1 face(s). Reference:; Face< 1 >. Type:; Apply torque. Value:; 1 N.m.. Torque-2; Entities:; 1 face(s). Reference:; Face< 1 >. Type:; Apply torque. Value:; 1 N.m.. Torque-3; Entities:; 1 face(s). Reference:; Face< 1 >. Type:; Apply torque. Value:; 1 N.m.. Torque-4; Entities:; 1 face(s). Reference:; Face< 1 >. Type:; Apply torque. Value:; 1 N.m..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 10.361 mm. Tolerance; 0.518051 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 21826. Total Elements; 11009. Maximum Aspect Ratio; 19.259. % of elements with Aspect Ratio < 3; 92.8. Percentage of elements with Aspect Ratio > 10; 0.109. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:04. Computer name:; JOSH-G15.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.20706; 0.00578785; -14.8168; 14.8183. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.0245838; 0.0809659; 0.0185621; 0.0866279. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 4.024e-03N/mm^2 (MPa) Node: 9597; 4.298e+00N/mm^2 (MPa) Node: 18244. AutoRecover Of ShelledModel1-Static 5-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 259; 5.889e-02mm Node: 18935. AutoRecover Of ShelledModel1-Static 5-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 8.030e-08 Element: 9586; 1.216e-05 Element: 2861. AutoRecover Of ShelledModel1-Static 5-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. AutoRecover Of ShelledModel1-Static 5-Displacement-Displacement1{1}.

# Conclusion