|  | Simulation of ShelledModel1 Date: April 1, 2023 Designer: Solidworks Study name: Static 5 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 11. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: ShelledModel1 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror4; Solid Body; Mass:8.72198 kg Volume:0.00111137 m^3 Density:7,847.94 kg/m^3 Weight:85.4754 N; C:\Users\westy\OneDrive\Documents\SubFrame Documents\Frame_Documents\ShelledModel1.SLDPRT Mar 30 12:43:40 2023.

# Study Properties Study name; Static 5. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\westy\OneDrive\Documents\SubFrame Documents\Frame_Documents).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/mm^2 (MPa).

# Material Properties Model Reference; Properties; Components. Name:; AISI 4130 Steel, normalized at 870C. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 460 N/mm^2. Tensile strength:; 731 N/mm^2. Elastic modulus:; 205,000 N/mm^2. Poisson's ratio:; 0.285. Mass density:; 7.85 g/cm^3. Shear modulus:; 80,000 N/mm^2.; SolidBody 1(Mirror4)(ShelledModel1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 4 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.0191936; 160.849; 0.00788128; 160.849. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Distributed Mass-1; Entities:; 4 face(s). Type:; Displacement (Direct transfer). Coordinate System:; Global cartesian coordinates. Translation Values:; ---, ---, --- mm. Rotation Values:; ---, ---, --- deg. Reference coordinates:; 0 0 0 mm. Remote Mass:; 7.7 kg. Moment of Inertia:; 0,0,0,0,0,0 kg.m^2. Components transferred:; NA.. Gravity-1; Reference:; Top Plane. Values:; 0 0 -9.81. Units:; m/s^2..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 10.361 mm. Tolerance; 0.518051 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 21826. Total Elements; 11009. Maximum Aspect Ratio; 19.259. % of elements with Aspect Ratio < 3; 92.8. Percentage of elements with Aspect Ratio > 10; 0.109. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:05. Computer name:; JOSH-G15.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.0191936; 160.849; 0.00788128; 160.849. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.137809; 56.7884; -0.0657987; 56.7886. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.890e-03N/mm^2 (MPa) Node: 11640; 7.336e+00N/mm^2 (MPa) Node: 12816. ShelledModel1-Static 5-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 259; 6.042e-02mm Node: 993. ShelledModel1-Static 5-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.681e-08 Element: 986; 2.281e-05 Element: 2566. ShelledModel1-Static 5-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 3.000e+00 Node: 1; 3.000e+00 Node: 1. ShelledModel1-Static 5-Factor of Safety-Factor of Safety1.

# Conclusion